# SOLSA ID 2A

Maintenance of the hyperspectral cameras VNIR AND SWIR

Céline Rodriguez – Beate Orberger

Date 31 March 2017

SOLSA VNIR and SWIR cameras were installed at TFS-INEL the 30th March with one-day training.

We present here information, which are important to maintain the cameras in laboratory but also on mine sites in particular important for future installation in New Caledonia.

This text will be regulary up-dated with new information.

SPECIM provides 2 years gratis technical support. After these two years a contract will be proposed. SPECIM answers all questions via a technical support platform.

The technical description of the two cameras is available on the CLOUD

The mail contact in Finland is Ana Aranda (ana.aranda@SPECIM.FI)

Preliminary up-set of the VNIR and SWIR cameras (31st March 2017)

GENERAL INFORMATIONS

VNIR

Model: SPECIM FX10 Serial: 1200039 Date: 20.3.2017 Guarantee: 2 years for technical support Acquisition software: Processing software: ENVI Spectral range: 400 – 1000 nm Weight: 1,6 kg

The FX10 camera is robust against humidity and dust and do not require specific maintenance. Be careful to not put your fingers on the optics. For cleaning alcohol or acetone and a soft tissue are required.

The focus is set manually. It depends on the operator. Use the following appropriate keys:

The white reference should be placed at the same level as the surface of the analyzed sample. We may buy also the grey reference for the darker lithologies of laterite drill cores. The offer should be sent to Monique.

VNIR do not need recalibration as no detector damages occurs as with SWIR.

SWIR

Model: SPECIM SWIR3 Serial: SN431048 Date: 1.2.2017 Guarantee: 2 years for technical support Acquisition software: Processing software: ENVI Spectral range: 900 – 1700 nm Weight: 12 kg

The weight of this camera is explained by the presence of optical stabilization, heat pump and optics larger than in the VNIR.  The camera is temperature stabilized.

If the heat pump is broken, return the camera to SPECIM.

The focus is set manually. Before accessing the optics, unscrew the protecting cover. Be careful to not put your fingers on the optics. For cleaning, alcohol or acetone are required.

The camera needs 5 to 10 minutes to be stabilized, prior to scanning. The reference should be analysed for each drill core.

HALOGENS

Six individual halogens are mounted to insure optimal lighting and reflectance.

Provisional up-set of the 6 halogens (15W/lamp): 3 on one side of the gutter and 3 on the other side of the gutter, placed at about 45°: not optimal for full reflectance recovery.

The halogens must be placed on either side of the sample so that there is not shadow zone.

The incident light makes it possible to measure the reflectance.

BAD PIXEL GENERATION by the detector of SWIR camera will increase with time.

This imply a detector refreshing and recalibration at the manufacturer SPECIM (about 2 K€ + transport). In case continuous measurements with SWIR should be performed, a second SWIR camera should be on-site.